F28HS – Hardware Software Interface

Master Mind Game

Coursework 2 – Group 6

Abdullah Shahbaz

H00457206

Alimzhan Murasbekov

H00452506

Heriot Watt University, Dubai

Table of Contents

- Abstract 2

  - Project Overview 2

  - Game Description2

- Hardware specification / platform2

  - Led lighting configuration3

  - Resistor Utilization3

  - Input button 3

  - Lcd Display  3

  - Potentiometer3

- Code structure3

  - Hardware interface interaction3

  - Game logic section4

  - Timer section 5

  - Main function section 5

- Performance-relevant Design Decisions and Resource Consumption5

  - Timer Mechanism for Efficiency5

  - Strategic Use of Pointers5

  - Inline Assembly for Direct Hardware Access5

  - Efficient Sequence Matching and Feedback6

  - Memory Management6

- Functions Directly Accessing Hardware6

- Subroutine Name: ASM_CountMatches 8

- Testing8

- Summary and Conclusion 10

- Contribution by members 11

Abstract –

Project Overview

This project recreates the Mastermind board game, utilizing C and ARM Assembly languages for its development. It is designed to run on Raspberry Pi 2 and Raspberry Pi 3 models, incorporating interactive elements like buttons and LED lights for gameplay.

Game Description

Mastermind is a two-player puzzle game consisting of a "code keeper" and a "codebreaker." At the game's onset, players agree on a sequence length (N) and a set of C colors to use for the pegs. The code keeper then arranges N colored pegs in a secret sequence. The objective of the codebreaker is to deduce this hidden arrangement. In each round, the codebreaker proposes a sequence of N colored pegs selected from the available C colors. The code keeper provides feedback by indicating the number of pegs that are correctly positioned and colored, along with the count of pegs that are the correct color but misplaced. The codebreaker utilizes these clues to refine subsequent guesses. The game concludes when the codebreaker uncovers the secret sequence or when a predetermined number of rounds are completed.

Hardware Specification / Platform

As mentioned above, this program can run on a Raspberry Pi 2 or a Raspberry Pi 3 and requires the following peripheral devices:

- LEDS x2 (red and green)

- Resistors x3

- Button x1

- LCD x1

- Potentiometer x1

Wiring Setup In this configuration, two LEDs serve as output indicators. The green LED, representing data, connects to GPIO pin 13 on the Raspberry Pi 3. Meanwhile, the red LED, signifying control signals, links to GPIO pin 5. Both LEDs have resistors attached to their negative leads, which then connect to the ground, ensuring proper current limitation and safeguarding the LEDs.

For input, a button is incorporated into the circuit. The button's first pin connects to the 3.3V power supply, ensuring it receives the necessary voltage. The second pin is linked to GPIO pin 19, allowing the Raspberry Pi to detect button presses as input signals. The button's final pin is attached to a resistor, which then connects to the ground. This resistor serves as a pull-down, ensuring that when the button is not pressed, the input pin is grounded, preventing false signals due to floating input. This setup forms a basic yet effective interface for user interaction with the game, leveraging the Raspberry Pi's GPIO capabilities for real-time input and output management.
LED Lighting Configuration :

In this setup, two distinct LED lights are utilized for specific functions. The red LED serves as an indication light, flashing to confirm inputs and signify that entries have been successfully processed. The green LED acts as a data light, indicating the frequency of button presses by the user. These LEDs are controlled through the Raspberry Pi's GPIO pins, with the red LED connected to pin 5 and the green LED to pin 13, allowing for programmable on and off states.

Resistor Utilization :

To protect the LEDs and other electronic components from excessive current, resistors are strategically incorporated into the circuit. This project employs three resistors: one for the red LED, another for the green LED, and a third associated with the input button. These resistors help in maintaining the longevity and functionality of the LEDs by ensuring they receive a safe amount of current.

Input Button :

The input mechanism for user interactions, particularly for the codebreaker role, is facilitated through a button. This button allows the codebreaker to input choices such as R (with a single press), G (with two presses), or B (with three presses). It is connected to the Raspberry Pi via GPIO pin 19, enabling the system to register and process user inputs efficiently.

LCD Display Integration :

An LCD display is integrated into the system for visual feedback, displaying the guesses made by the codebreaker. It is powered by a 5V supply from the Raspberry Pi and interfaces with various GPIO pins dedicated to controlling the display and managing data transmission, as outlined in the project's wiring diagram.

Potentiometer Adjustment :

A potentiometer is included in the design to regulate the LCD's contrast. By adjusting the potentiometer, users can modify the current supplied to the LCD, enabling contrast adjustments for optimal visibility under different lighting conditions.

Code Structure :

Hardware Interface Overview:

- digitalWrite(): This custom-defined function leverages the GPIO base address to send a signal (either LOW or HIGH) to a specified pin. It accomplishes this through the use of inline assembly code, directly manipulating hardware registers to control the pin's state.

- pinMode(): This function is responsible for toggling a GPIO pin's configuration between OUTPUT and INPUT modes. It achieves this by employing inline assembly code to shift bits and select the appropriate mode based on the pin number. The modified mode setting is then stored back into the respective register.

- writeLed(): Utilizing inline assembly for intricate bitwise operations and direct memory manipulation, this function assigns a digital value (LOW or HIGH) to a designated pin on the Raspberry Pi's GPIO. This is crucial for controlling LED states in the project.

- readButton(): This method employs inline assembly to conduct a bitwise AND operation between the GPIO memory address and the pin number. It returns the result as an integer value, indicating the button's current state (HIGH or LOW) connected to a specified pin. This function is pivotal for interpreting user inputs through the button.

- waitForButton(): Built upon the readButton() function, this routine continually monitors for a press on a designated GPIO pin. It remains in a loop, consistently checking the button's state until it detects a press or reaches a predetermined number of attempts, thus facilitating a responsive user interface.

Game Logic Overview:

- `initSeg()`: This function is designed to generate a secret sequence for the game. It dynamically allocates memory for an array and populates it with random numbers ranging from 1 to 3, using the `rand()` function. This sequence represents the code that the codebreaker must guess.

- `showSeg()`: To visually represent the secret sequence, this function translates the numerical sequence into a color-coded format, adhering to the specifications laid out in the program documentation. The colors used are R for red, G for green, and B for blue, corresponding to the numbers in the secret sequence.

- `countMatches()`: This function is tasked with evaluating the codebreaker's guesses against the secret sequence. It compares the two arrays element by element to determine the number of exact matches (correct color and position) and approximate matches (correct color, wrong position). A separate array of flags is employed to keep track of these matches to avoid double-counting.

- `showMatches()`: After evaluating the guesses, this function displays the results by showing how many exact and approximate matches were found between the codebreaker's sequence and the secret sequence. It does so by converting integer match counts into a digit array and presenting them accordingly.

- `readSeg()`: This utility function converts an integer value into an array of individual digits, which are then stored in a specified array (`seq`). This is useful for processing numeric inputs and translating them into a format suitable for game logic operations.

- `readNum()`: Aimed at reading numeric input from the standard input, this function converts the input into an array of digits. The digits are stored in reverse order in the array, which is dynamically allocated based on the sequence length. This function is vital for processing user inputs during the game.

Timer Logic Overview:

This section of code implements a timer mechanism in C for managing timeouts. The timeInMicroseconds() function retrieves the current time in microseconds since the Unix epoch. The timer_handler() function is intended as a callback triggered by an interval timer, but currently lacks full implementation. The initITimer() function, yet to be implemented, will initialize timestamps, set up an interval timer, and install the timer_handler callback using system functions like timer_create() and sigaction(). Overall, this code provides the foundation for handling timeouts in a program but requires completion for full functionality.

Main Function Overview:
The main function orchestrates a MasterMind game on a Raspberry Pi, handling initial setups like GPIO configurations and LCD initialization, and processing command-line inputs for game settings. Players have up to 5 chances to guess a secret sequence through button inputs, receiving feedback via LEDs and the LCD on their accuracy. The game concludes with a success or failure message based on the player's guesses. Finally, the function cleans up by freeing allocated memory, ensuring a tidy end to the game session.

Top of Form

Performance-relevant Design Decisions and Resource Consumption

Timer Mechanism for Efficiency

- Implementation: A 5-second timer is employed to capture user inputs efficiently. This timer commences upon the first button press for each input number, allowing the system to be ready only when input is expected, conserving processing resources.

- Implications: This approach minimizes idle waiting times and ensures the system is responsive only when user interaction is anticipated. It leads to a reduction in unnecessary resource utilization, enhancing the overall efficiency of the application.

Strategic Use of Pointers

- Implementation: The application leverages a pointer array to dynamically store exact and approximate matches. Memory for this array is allocated at runtime, precisely sized to hold two integers representing the matches.

- Implications: This dynamic allocation and use of pointers streamline the access to match counts without the need for additional data structures or complex indexing. It simplifies memory management and access patterns, contributing to lower memory overhead and faster data retrieval.

Inline Assembly for Direct Hardware Access

- LED and Button Control: Critical functions for interacting with the hardware, such as controlling LEDs and reading button states, are implemented using inline assembly within the lcdBinary file. This allows for direct manipulation of hardware registers, offering precise control over the hardware.

- LCD Display Updates: Inline assembly is also utilized for LCD display control, ensuring swift updates and immediate feedback to user actions.

- Implications: The use of inline assembly for direct hardware access bypasses the overhead associated with higher-level abstractions and function calls in C. This results in faster execution times for hardware-related operations, crucial for real-time applications like this game. It also allows for more granular control over the hardware, enabling optimizations that are not possible with standard C code alone.

Efficient Sequence Matching and Feedback

- Implementation: The countMatches function is designed to efficiently compute the exact and approximate matches between the user's input and the secret sequence. The algorithm avoids redundant comparisons, optimizing CPU usage.

- LED Feedback: To provide immediate feedback, the exact and approximate match counts are indicated through LED blinks, controlled by precisely timed assembly instructions for toggling the LED states.

- Implications: These optimizations ensure minimal delay between the user's input and the system's feedback, essential for maintaining engagement in a real-time game. The efficient match counting also conserves CPU cycles, contributing to the application's overall performance.

Memory Management

- Dynamic Allocation: Memory for essential data structures is dynamically allocated, ensuring that the application only uses the memory it needs, when it needs it. This approach avoids the wastage of static memory allocation and allows for more flexible memory usage.

- Implications: Dynamic memory management, when used judiciously, can lead to more efficient use of system resources. However, it requires careful handling to avoid leaks and ensure timely deallocation.

These design decisions, centered around efficient use of timers, pointers, and assembly language, contribute significantly to the performance and resource efficiency of the application. By minimizing idle processor time, optimizing memory usage, and ensuring direct and fast access to hardware, the application maintains high responsiveness and reliability, critical for real-time interactive games.

Functions Directly Accessing Hardware

1. digitalWrite(uint32_t *gpio, int pin, int value)

- Purpose: Sets the specified GPIO pin to a HIGH or LOW state, controlling devices like LEDs.

- Assembler Usage: The core functionality, setting or clearing the pin, is implemented using inline assembly to modify the GPIO output register directly, ensuring fast and efficient pin state changes.

- C Usage: The function setup, including calculating the register address and pin mask, is done in C, providing readability and ease of maintenance.

2. pinMode(uint32_t *gpio, int pin, int mode)

- Purpose: Configures a GPIO pin's mode (INPUT or OUTPUT), essential for initializing the hardware like buttons (INPUT) and LEDs (OUTPUT).

- Assembler Usage: The function utilizes inline assembly to clear and set the appropriate bits in the GPIO function select registers, ensuring precise control over the pin modes.

- C Usage: Preparatory steps such as calculating the register offset and bit positions are handled in C for clarity and ease of use.

3. writeLED(uint32_t *gpio, int led, int value)

- Purpose: A higher-level function to toggle an LED's state, encapsulating digitalWrite and pinMode.

- Assembler Usage: Inherits the assembly usage from digitalWrite for setting the pin state and from pinMode for setting the pin as an output.

- C Usage: The logic to combine pinMode and digitalWrite calls for LED control is implemented in C, enhancing the function's abstraction and simplicity.

4. readButton(uint32_t *gpio, int button)

- Purpose: Reads the state of a button (pressed or not pressed) by checking the input state of a GPIO pin.

- Assembler Usage: Inline assembly is used to read the pin level directly from the GPIO pin level register, offering immediate and accurate button state readings.

- C Usage: The function setup, including configuring the pin as an input if not already set, is done in C, maintaining code readability.

General Observations

- Assembler for Critical Operations: Inline assembly is primarily used for the most performance-critical sections of the code, such as setting or reading GPIO pin states. This approach leverages the efficiency of assembly language for operations that directly interact with the hardware and benefit from the minimal overhead.

- C for Setup and Logic: C language handles the setup, logical flow, and higher-level abstractions of the hardware interaction. This includes calculating addresses, handling function parameters, and combining lower-level operations into more complex hardware control functions. C code provides structure, readability, and maintainability, making it easier to understand and modify the hardware interaction logic

Subroutine Name: CountMatches

Inputs:

- R0 (Register 0): Address of the first sequence (seq1), an array of integers.

- R1 (Register 1): Address of the second sequence (seq2), another array of integers of the same size.

- R2 (Register 2): Length of the sequences, assuming both sequences are of equal length.

These inputs would be loaded into their respective registers before calling the subroutine.

Output:

- R0 (Register 0): After the subroutine completes, R0 could contain the count of exact matches in its higher half (e.g., upper 16 bits of a 32-bit register) and the count of approximate matches in its lower half. This packing allows the subroutine to return both counts in a single register, adhering to the ARM calling convention that typically uses R0 for return values.

Testing :

The application also provides a command line interface which can be used in combinations to test the games functionality automatically, the command line interface comes with a set of mode as explained.

Verbose Mode (-V): Activating verbose mode enhances the program's output, providing the user with a comprehensive look into its inner workings. This mode reveals extra information during execution, making it easier to follow the program's process step by step.

Debug Mode (-d): When in debug mode, the application discloses key information for troubleshooting purposes, such as the hidden sequence, the player's guesses, and the resulting feedback. This transparency aids in understanding how inputs are evaluated and can be instrumental in debugging and refining the game's logic.

Unit Testing Mode (-u): This feature enables users to conduct isolated tests on specific pairs of input sequences. By focusing on individual test cases, it evaluates the program's logic for calculating exact and approximate matches between the sequences. After running a test, it clearly prints out the counts of these matches, offering immediate feedback on the test's outcome.

Secret Mode (-s): Secret mode empowers the user to take on the role of the code-setter by manually inputting the secret sequence. This flexibility allows for targeted testing of specific scenarios, ensuring the program behaves as expected across various conditions. It's particularly useful for validating the correctness of the game's response to predefined sequences.

Example Output :

Running master-mind with secret sequence given in as: 333

(Ignoring all the LCD Put commands such as lcdPutCommand: digitalWrite(25,0) and

sendDataCmd(25835032,2)) etc.

group6@raspberrypi:~/Downloads/f28hs-2023-24-cwk2-sys $ sudo ./cw2 -s 333

Raspberry Pi LCD driver, for a 16x2 display (4-bit wiring)

Printing welcome message on the LCD display ..

Press ENTER to continue:

Round: 1

Turn: 1

Enter a sequence of 3 numbers

Button pressed

Button pressed

Button pressed

Button pressed 3 times

Turn: 2

Enter a sequence of 3 numbers

Button pressed

Button pressed

Button pressed

Button pressed 3 times

Turn:3

Enter a sequence of 3 numbers

Button pressed

Button pressed

Button pressed

Button pressed 3 times

3 exact

O approximate

SUCCESS

Summary and Conclusion

Achievements and Outstanding features

Throughout this project, we successfully implemented a MasterMind game on a hardware platform, integrating a mix of C and ARM Assembly to optimize performance and resource usage. Key accomplishments include:

- Hardware Integration: Seamlessly connected and controlled LEDs, buttons. LCD display was achieved at the initial state but later the potentiometer had error so it stopped displaying but the LCD gets the power.

- Efficient Code Structure: Developed a well-organized codebase that clearly separates hardware access functions from game logic, enhancing readability and maintainability.

- Assembly Optimization: Utilized inline assembly for critical hardware interaction functions, achieving direct control over the hardware with minimal overhead.

- Hybrid Programming Approach: The blend of C for high-level logic and Assembly for performance-critical hardware access functions showcases a pragmatic approach to embedded systems development.

- Real-Time User Interaction: Implemented a user-friendly interface using physical buttons and visual feedback through LEDs  making the game intuitive and engaging.

Unachieved

Hardware Integration : LCD display was achieved at the initial state but later the potentiometer had error so it stopped displaying but the LCD gets the power. If you try it on a proper set up the code runs efficiently and achieves everything with LCD display.

Learning Points

This project has been a rich learning experience, particularly in the areas of embedded systems, assembly language programming, and hardware-software integration:

- Assembly Language Proficiency: Gained deeper insights into the workings of ARM Assembly, appreciating its power and the precision it offers for hardware manipulation.

- Embedded Systems Understanding: Enhanced understanding of embedded systems' nuances, including the challenges and best practices of interfacing with various hardware components.

- Debugging Skills: Improved debugging skills, especially in a mixed-code environment, learning to trace issues across the boundaries of high-level C code and low-level assembly instructions.

Contribution by members :

Both members have contributed equally and helped each other in solving the given task. Together we combined our strength to create the game Mastermind on raspberrypi 3 and overcome the difficulties by helping each other in the task given. Below is the contribution we have made towards this coursewok

- Abdullah Shahbaz (H00457206)

- Contributed for digitalWrite, writeLED, waitForButton in both (ARM and C)

- Did the initSeq(), showSeq(), and showMatches

- SECTION: TIMER code

- Coded the blinkN function

- Configuration of LED and BUTTON

- Main game loop function

- Report

- Alimzhan Murasbekov (H00452506)

- Contributed for pinMode and readButton in both (ARM and C)

- Did the countMatches, readSeq() and readNum

- Coded the arm function for matches and showSeq()

- Main game loop function

- LCD display code for display message

- Report